# Appendix A: Metadata for SWAT Baseline Data.

Table 1. Metadata of the SWAT catchment outline data of SA.

| Metadata type | Digital elevation model | Catchment outlines |  | Catchment inlets and outlet locations | DWS monitoring station locations |
| --- | --- | --- | --- | --- | --- |
| Title | SRTM_DEM_90m | Catchments_tertiary_ buffer100m | Catchments_quaternary_ buffer100m | Catchment_inlets_outlets | DWS_monitoringstations_ locations |
| Date created | 2012 | 2018 | 2018 | 2022 | 2022 |
| Purpose | Hydrologically improved DEM for SA | Tertiary catchment shapefile with 100m buffer to identify area of interest | Quaternary catchment shapefile with 100m buffer to identify area of interest | Tertiary and quaternary catchment outlets and inlets locations | DWS monitoring stations locations |
| Description of base datasets | Global SRTM DEM https://www.usgs.gov/centers/eros/science/usgs-eros-archive-digital-elevation-shuttle-radar-topography-mission | Tertiary catchment map available at DWS Spatial Data & Application Portal https://gia.dws.gov.za/portal/home | Quaternary catchment map available at DWS Spatial Data & Application Portal https://gia.dws.gov.za/portal/home | Flow paths derived from flow accumulation of hydrologically improved DEM for SA (where both entering and exiting flow paths overlay with catchment boundaries) | DWS monitoring stations locations available at DWS Resource Quality Information Services Portal https://www.dws.gov.za/iwqs/gis_data/ |
| Limitations and assumptions | Overall accuracy 90% | Outlines based on Hydrologically improved DEM for SA of Weepener et al. (2012) | Outlines based on Hydrologically improved DEM for SA of Weepener et al. (2012) | Point locations based on Hydrologically improved DEM for SA of Weepener et al. (2012) | N/A |
| Place keywords | National | National | National | National | National |
| Citation | WRC report 1908/1/11, Weepener et al. (2012) | DWS (2022a) | DWS (2022a) | N/A | DWS (2022b) |
| Spatial representation | Format: GRID Resolution: 90 m Spatial Reference: WGS_1984_Albers | Format: Shapefile Spatial Reference: WGS_1984_Albers | Format: Shapefile Spatial Reference: WGS_1984_Albers | Format: Shapefile Spatial Reference: WGS_1984_Albers | Format: Shapefile Spatial Reference: WGS_1984_Albers |

Table 2. Metadata of the South African National Land Cover datasets.

| Metadata type | National land cover map 2013/14 | National land cover map 2018 | National land cover map 2020 |
| --- | --- | --- | --- |
| Title | NLC2013_SWAT | NLC2018_SWAT | NLC2020_SWAT |
| Date created | 2013-2014 | 2018 | 2020 |
| Purpose | 2013/14 national land cover grid of SA, of which the grid values are reclassified to match the SWAT land cover codes. | 2018 national land cover grid of SA, of which the grid values are reclassified to match the SWAT land cover codes. | 2020 national land cover grid of SA, of which the grid values are reclassified to match the SWAT land cover codes. |
| Description of base datasets | The South African National Land-Cover 2013/14 dataset with 72 land cover classes has been generated from 30 m multi-seasonal Landsat 8 imagery. Available from https://www.environment.gov.za/ projectsprogrammes/egis_landcover_datasets | The South African National Land-Cover 2018 dataset with 73 land cover classes has been generated from 20 m multi-seasonal Sentinel 2 satellite imagery. Available from https://www.environment.gov.za/ projectsprogrammes/egis_landcover_datasets | The South African National Land-Cover 2020 dataset with 73 land cover classes has been generated from 20 m multi-seasonal Sentinel 2 satellite imagery. Available from https://www.environment.gov.za/ projectsprogrammes/egis_landcover_datasets |
| Limitations, assumptions | Overall map accuracy 82% | Overall map accuracy 91% | Overall map accuracy 85% |
| Place keywords | National | National | National |
| Citation | SANLC (2014) | SANLC (2018) | SANLC (2020) |
| Spatial representation | Format: GRID Resolution: 30 m Spatial Reference: WGS_1984_Albers | Format: GRID Resolution: 20 m Spatial Reference: WGS_1984_Albers | Format: GRID Resolution: 20 m Spatial Reference: WGS_1984_Albers |

LU_landcover2020.csv and  LU_landcover2020.txt – comprises the user lookup table that links abovementioned 2020 national land cover grid with the SWAT land cover codes;

LU_landcover2018.csv and  LU_landcover2018.txt – comprises the user lookup table that links abovementioned 2018 national land cover grid with the SWAT land cover codes;

LU_landcover2013.csv and  LU_landcover2013.txt – comprises the user lookup table that links abovementioned 2013 national land cover grid with the SWAT land cover codes.

Table 3. Metadata of the soil map that was derived from the ARC Land Type Database of SA.

| Metadata type | Soil map (without soil attribute data) | Soil database table (with soil attribute data) |
| --- | --- | --- |
| Title | Usersoil_SA | Format 1: Usersoil_SA.accdb Format 2: Usersoil_SA.csv |
| Date created | 1972-2006 | 2022 |
| Purpose | Land Type shapefile of SA illustrating locations of land type (soil combinations). | Soil attribute data required by the SWAT model. |
| Description of base datasets | A national soil, climate and terrain database in digital format, developed by the Agricultural Research Council in Southa Africa, including polygons that reflects the dominant soils in each land type by percentage. | Microsoft Access database (accdb) and comma-separated value (CSV) tables with soil attribute data for each of the 16556 Land Types of SA, including textural and soil hydraulic parameter values for two soil layers. |
| Limitations and assumptions | Data cannot indicate exactly where within each Land Type the various soils occur. It is thus not a substitute for a detailed soil map. | Several assumptions (pedotransfer functions) (see le Roux et al., 2022) had to be made to assign soil parameter values to Land Types at a national scale. |
| Place keywords | Catchments within South African borders | Catchments within South African borders |
| Citation | ARC Land Type Database (2012) | Le Roux et al. (2022) |
| Spatial representation | Format: Shapefile Spatial scale: useable scale 1:250,000 Spatial Reference: WGS_1984_Albers | Format 1: Microsoft Access database table Format 2: Microsoft Excel csv table |

LU_usersoil_SA.txt and LU_usersoil_SA.csv – comprises the user lookup table that links abovementioned Land Type grid with relevant database table with soil attribute data for each Land Type of SA.

Table 4. Metadata of the weather statistics (WGN) files of SA.

| Metadata type | Weather statistics (WGN) 1981-2000 | Weather statistics (WGN) 2001-2020 |
| --- | --- | --- |
| Title | Format 1: wgenuser1981-2000.accdb Format 2: wgenuser1981-2000.csv | Format 1: wgenuser2001-2020.accdb Format 2: wgenuser2001-2020.csv |
| Date created | 2021 | 2021 |
| Purpose | Weather statistics database table with 11 WGN files containing the weather generator (WGN) statistics from 1981-2000 of 11 stations. | Weather statistics database table with 12 WGN files containing the WGN statistics from 2001-2020 of 12 stations. |
|  | WGN input files consists of several weather statistics needed by SWAT to generate representative daily climate data for simulated catchments in two instances: when the user specifies that simulated weather will be used or when measured data is missing. |  |
| Description of base datasets | Agroclimatology Database, Agricultural Research Council-Soil, Climate and water including precipitation, temperature, solar radiation, relative humidity and wind speed. |  |
| Limitations and assumptions | Daily weather data are not provided. SWAT users have to obtain their own daily climate data. 12 WGN files were created by acquiring and interpreting climate data from ARC (2021) weather stations in different climate zones in SA. The completeness of climate data was the most important consideration for selecting weather stations. The station in Hobhouse (30673), did not have enough data to calculate statistics for the period 1981-2000, and were excluded in the 1981-2000 dataset. Mechanical stations only measured sunshine hours and not solar radiation. Therefore, solar radiation data for the period 2001-2020 were used for the calculation of all WGN files. Furthermore, the mechanical stations measured wind run, which was converted to wind speed for the purposes of this project. |  |
| Place keywords | National | National |
| Citation | N/A | N/A |
| Spatial representation | Format 1: Microsoft Access database table Format 2: Microsoft Excel csv table | Format 1: Microsoft Access database table Format 2: Microsoft Excel csv table |

References

ARC Land Type Database, 2012. Land Types of South Africa: Maps (69 sheets) and Memoirs (39 books). Agricultural Research Council -Soil, Climate and Water, Pretoria, South Africa.

DWS. 2022a. Spatial Data & Application Portal. Department of Water and Sanitation: Pretoria, South Africa. Available from: https://gia.dws.gov.za/portal/home. Accessed 03 March 2022.

DWS. 2022b. Resource Quality Information Services Portal. Department of Water and Sanitation: Pretoria, South Africa. Available from: https://www.dws.gov.za/iwqs/gis_data. Accessed 03 March 2022.

le Roux, J.J., Mararakanye, N., Mudaly, L., Weepener, H.L., van der Laan, M., 2023. Development of a South African national input database to run the SWAT model in a GIS. WRC report 3053/1/22. Water Research Commission, Pretoria, South Africa. ISBN 978-0-6392-0454-3.

SANLC, 2014. National Land Cover Data of South Africa. 1990 and 2013-14. Department of Environmental Affairs: Pretoria, South Africa. Available from https://www.environment.gov.za/projectsprogrammes/egis_landcover_datasets.

SANLC, 2018. National Land Cover Data of South Africa 2018. Department of Environmental Affairs: Pretoria, South Africa. Available from https://www.environment.gov.za/projectsprogrammes/egis_landcover_datasets.

SANLC, 2020. National Land Cover Data of South Africa 2020. Department of Environmental Affairs: Pretoria, South Africa. Available from https://www.environment.gov.za/projectsprogrammes/egis_landcover_datasets.

Weepener, H.L., van den Berg, H.M., Metz, M., Hamandawana, H., 2012. The development of a hydrologically improved Digital Elevation Model and derived products for South Africa based on the SRTM DEM. WRC report 1908/1/11. Water Research Commission, Pretoria, South Africa. ISBN 978-1-4312-0217-1.