Supplementary Information

Modelling ecological risks of antiretroviral drugs in the environment

P. Ngwenya, N. Musee1

Emerging Contaminants Ecological and Risk Assessment (ECERA) Research Group, Department of Chemical Engineering, University of Pretoria, Private Bag X20, Hatfield, 0028, Pretoria, South Africa

1. HIV, ARVs and sanitation statistics

Figure S1: The population percentage in SSA countries with high HIV prevalence [1].

2. Environmental occurrence of ARVs

Table S1: Measured environmental concentrations (MECs) of ARVs from WWTP influent or effluent samples reported globally based on widely used treatment technologies.

| ARV drug | Concentration (μg/L) |  | Country |  | Reference |
| --- | --- | --- | --- | --- | --- |
|  | Influent | Effluent |  |  |  |
| Abacavir | 14 | < LOD | South Africa |  | [2] |
| Abacavir | 3.5 | < LOD | South Africa |  | [2] |
| Abacavir | 0.14 | < LOQ | Germany |  | [3] |
| Abacavir | 0.086 |  | Germany |  | [4] |
| Abacavir | 0.225 | < LOQ | Germany |  | [5] |
| Atazanavir | 1.4 | 0.74 | South Africa |  | [2] |
| Atazanavir | 0.064 | 0.078 | South Africa |  | [2] |
| Darunavir | 43 | 17 | South Africa |  | [2] |
| Darunavir | 0.92 | 0.35 | South Africa |  | [2] |
| Efavirenz | 0.78 | 0.11 | Kenya |  | [6] |
| Efavirenz | 0.46 | 0.1 | Kenya |  | [6] |
| Efavirenz | 1.02 | 0.11 | Kenya |  | [6] |
| Efavirenz | 140.4 |  | South Africa |  | [7] |
| Efavirenz | 34 | 20 | South Africa |  | [2] |
| Efavirenz | 34 | 34 | South Africa |  | [2] |
| Efavirenz | 24 | 33 | South Africa |  | [2] |
| Efavirenz | 1.37 | 3.63 | South Africa |  | [8] |
| Efavirenz | 26.3 | 37.3 | South Africa |  | [8] |
| Efavirenz | 1.67 | 3.27 | South Africa |  | [8] |
| Efavirenz | 1.02 |  | South Africa |  | [8] |
| Efavirenz | 2.22 | 3.64 | South Africa |  | [8] |
| Efavirenz | 17.4 | 7.1 | South Africa |  | [9] |
| Efavirenz | 15.4 | 1.93 | South Africa |  | [10] |
| Emtricitabine | 172 | 4.23 | South Africa |  | [10] |
| Emtricitabine | 0.051 |  | Germany |  | [4] |
| Emtricitabine | 0.98 | 0.14 | Germany |  | [3] |
| Emtricitabine | 3.10 | 0.22 | South Africa |  | [8] |
| Emtricitabine | 0.30 | 0.22 | South Africa |  | [8] |
| Emtricitabine | 1.47 | 0.35 | South Africa |  | [8] |
| Emtricitabine | < LOQ | 0.11 | South Africa |  | [8] |
| Indinavir | 0.59 | 0.042 | South Africa |  | [2] |
| Indinavir | 0.26 | 0.025 | South Africa |  | [2] |
| Indinavir | 0.3 | 0.04 | South Africa |  | [2] |
| Lamivudine | 0.37 | < LOQ | Germany |  | [3] |
| Lamivudine | 0.014 | 0.001 | Belgium |  | [11] |
| Lamivudine | 30.3 | 19.9 | Kenya |  | [6] |
| Lamivudine | 60.6 | 31.07 | Kenya |  | [6] |
| Lamivudine | 31.46 | 29.87 | Kenya |  | [6] |
| Lamivudine |  | 3.98 | Kenya |  | [12] |
| Lamivudine | 0.84 | < LOD | South Africa |  | [2] |
| Lamivudine | 1.9 | < LOD | South Africa |  | [2] |
| Lamivudine | 2.2 | 0.13 | South Africa |  | [2] |
| Lamivudine | 20.9 | < LOQ | South Africa |  | [10] |
| Lamivudine | 118.97 | 55.76 | Zambia |  | [13] |
| Lamivudine | 0.72 | < LOQ | Germany |  | [5] |
| Lopinavir |  | 0.13 | South Africa |  | [14] |
| Lopinavir | 1.3 | 3.8 | South Africa |  | [2] |
| Lopinavir | 1.2 | 1.9 | South Africa |  | [2] |
| Lopinavir | 2.5 | 3.8 | South Africa |  | [2] |
| Maraviroc | 0.082 | 0.039 | South Africa |  | [2] |
| Nevirapine |  | 1.357 | Kenya |  | [12] |
| Nevirapine | 0.85 | 1.03 | Kenya |  | [6] |
| Nevirapine | 3.3 | 2.11 | Kenya |  | [6] |
| Nevirapine | 2.08 | 2.03 | Kenya |  | [6] |
| Nevirapine | 0.011 | 0.0002 | Canada |  | [11] |
| Nevirapine | 0.64 | 1.72 | Zambia |  | [13] |
| Nevirapine | 0.67 | 0.54 | South Africa |  | [2] |
| Nevirapine | 2.1 | 1.9 | South Africa |  | [2] |
| Nevirapine | 2.8 | 1.4 | South Africa |  | [2] |
| Nevirapine | 0.681 | 0.764 | South Africa |  | [10] |
| Nevirapine | 2.1 | 0.35 | South Africa |  | [9] |
| Nevirapine | 0.022 | 0.032 | Germany |  | [5] |
| Raltegravir | 17 | 3.5 | South Africa |  | [2] |
| Raltegravir | 0.81 | 0.086 | South Africa |  | [2] |
| Raltegravir | 0.061 | < LOD | South Africa |  | [2] |
| Ritonavir | 0.09 |  | Switzerland |  | [15] |
| Ritonavir | 3.2 | 1.5 | South Africa |  | [2] |
| Ritonavir | 1.6 | 0.91 | South Africa |  | [2] |
| Ritonavir | 1.6 | 0.46 | South Africa |  | [2] |
| Saquinavir | 0.18 | < LOD | South Africa |  | [2] |
| Stavudine | 0.788 |  | South Africa |  | [14] |
| Stavudine |  | 0.413 | South Africa |  | [14] |
| Stavudine | 0.011 | < LOQ | Germany |  | [5] |
| Tenofovir | 0.25 | 0.17 | South Africa |  | [8] |
| Tenofovir | 0.10 | < LOQ | South Africa |  | [8] |
| Zidovudine | 0.116 | 0.132 | Belgium |  | [11] |
| Zidovudine | 0.31 | 0.098 | Germany |  | [5] |
| Zidovudine | 0.38 | 0.564 | Germany |  | [5] |
| Zidovudine |  | 0.97 | South Africa |  | [14] |
| Zidovudine |  | 0.22 | South Africa |  | [14] |
| Zidovudine | 66.59 | 37.14 | Zambia |  | [13] |
| Zidovudine |  | 0.513 | Kenya |  | [16] |
| Zidovudine | 0.062 | 0.037 | Finland |  | [16] |
| Zidovudine | 107 | 97.7 | South Africa |  | [10] |
| Zidovudine | 6.9 | 0.087 | South Africa |  | [2] |
| Zidovudine | 53 | 0.5 | South Africa |  | [2] |
| Zidovudine | 11 | 0.43 | South Africa |  | [2] |
| Zidovudine | 0.008 | 0.006 | France |  | [17] |
| Zidovudine | 13.27 | 0.09 | Kenya |  | [6] |
| Zidovudine | 12.1 | 0.09 | Kenya |  | [6] |
| Zidovudine | 20.13 | 0.11 | Kenya |  | [11] |
| Zidovudine | 0.39 | 0.15 | Germany |  | [6] |

< LOD/Q – Lower than the limit of detection/quantification

Table S2: Summary of the MECs on ARVs in surface water samples globally.

| ARV drug | Concentration (μg/L) | Country | Reference |
| --- | --- | --- | --- |
| Didanosine | 0.05 | South Africa | [18] |
| Efavirenz | 1.59 | Kenya | [6] |
| Lamivudine | 0.016 | Belgium | [11] |
| Lamivudine | 3.15 | Kenya | [19] |
| Lamivudine | 167.10 | Kenya | [6] |
| Lamivudine | 4.4 | Kenya | [12] |
| Lamivudine | 0.09 – 0.24 | South Africa | [14] |
| Lamivudine | 49.7 | Zambia | [13] |
| Lopinavir | 0.28 – 0.31 | South Africa | [14] |
| Nevirapine | 33.44 | Kenya | [19] |
| Nevirapine | 5.62 | Kenya | [19] |
| Nevirapine | 4.87 | Kenya | [12] |
| Nevirapine | 0.003 | Canada | [11] |
| Nevirapine | 0.24 – 1.48 | South Africa | [14] |
| Nevirapine | 0.21 | Zambia | [13] |
| Stavudine | 0.0013 – 0.0043 | Germany | [5] |
| Stavudine | 0.102 – 0 .778 | Kenya | [19] |
| Stavudine | 0.41 – 0.78 | South Africa | [14] |
| Tenofovir | 0.11 ± 2.0 | South Africa | [8] |
| Tenofovir | 0.25 | South Africa | [14] |
| Tenofovir | 0.16 – 0.19 | South Africa | [14] |
| Zalcitabine | 0.07 | South Africa | [14] |
| Zalcitabine | 0.03 | South Africa | [14] |
| Zidovudine | 0.0012 – 0.17 | Germany | [8] |
| Zidovudine | 0.22 – 0.62 | South Africa | [14] |
| Zidovudine | 0.05 | South Africa | [14] |
| Zidovudine | 0.0517 – 0.973 | South Africa | [14] |
| Zidovudine | 7.684 | Kenya | [12] |
| Zidovudine | 0.0183 | Kenya | [19] |
| Zidovudine | 0.05 – 17.4 | Kenya | [6] |
| Zidovudine | 0.117 | Belgium | [11] |
| Zidovudine | 1.28 | Zambia | [13] |
| Zidovudine | 0.022 – 0.03 | Germany | [3] |

Table S3: MECs of ARVs from groundwater samples globally.

| ARV drug | Concentration (μg/L) | Location | Reference |
| --- | --- | --- | --- |
| Abacavir | 0.01 | Germany | [4] |
| Lamivudine | 0.001 | Germany | [4] |
| Lamivudine | 0.025 | USA | [20] |
| Nevirapine | 0.013 | South Africa | [21] |
| Nevirapine | 0.027 | USA | [20] |
| Nevirapine | 1.6 | Kenya | [6] |
| Zidovudine | 0.03 | Kenya | [6] |

Table S4: MEC of ARVs in tap water samples globally.

| ARV drug | Concentration (μg/L) | Location | Reference |
| --- | --- | --- | --- |
| Abacavir | 0.08 | Germany | [3] |
| Emtricitabine | 0.04 | Germany | [3] |
| Lamivudine | 0.084 | Germany | [3] |
| Lamivudine | 0.004 | USA | [22] |
| Lamivudine | < 0.001 | Germany | [23] |
| Zalcitabine | 0.008 | South Africa | [14] |
| Zidovudine | 0.07 | South Africa | [14] |
| Zidovudine | 0.003 | Poland | [24] |
| Zidovudine | 0.02-0.03 | Kenya | [6] |

Figure 2: Global statistics of PLWHIV in 2021 based on Joint United Nations Programme on HIV/AIDS (UNAIDS) data (UNAIDS,2021).

(a)

(a)

(b)

(b)

Figure S3: Global statistics on the distribution of ARVs in the environment with (a) as measured environmental concentrations, and (b) number of detected ARVs in a given country.

Figure S4: Schematic illustration of the ARV flows in Eswatini both in the rural and urban settings via variant technical and natural systems. Most rural population has limited or no connection to WWTPs, and a portion of ARVs are released via runoff without any form of treatment process.

3. Setting of problem boundary

The country of focus in this work is the kingdom of Eswatini. Eswatini is a landlocked country in Sub-Saharan Africa (SSA), and shares boundaries with South Africa and Mozambique [25,26]. The country is third least developed globally with a land area of 17 200 km2 and a population of some 1 106 164 people [27]. It has a density of 80.0 per km2 with 50.8 % as females, and the rest as males [28]. The country has four administrative regions, namely: Hhohho, Manzini, Lubombo, and Shiselweni (Figure S5); which are further sub-divided into 55 constituencies [29]. About 62% of Eswatini’s population resides in the two urban regions, namely; Mbabane and Manzini, with  320 651 and 355 945 people respectively in 2017 [30].

Eswatini has the highest global HIV prevalence at some 27.2% of the total population, with steep gender parity of 32.5 and 20.4% for females and males, respectively [31,32]. Geographically, the Lubombo region had the highest HIV prevalence, and Hhohho region the least (Figure S5). The country over the last decade increased the rollout of ART programmes to manage the HIV pandemic, and this led to a reduction in the prevalence rate. For example, HIV prevalence according to the Swaziland HIV Incidence Measurement Survey (SHIMS) 1, and SHIMS 2 reports documented an overall decline of 1.6% from 2011 to 2016/17 where 2.9% were among males, and 0.7% on females [33].

Eswatini prescribes universal ART to all children aged five and below since 2015, and thereafter, adopted the revised WHO guidelines of October 2016 [34]. The guidelines promoted the initiation of the “Test and Start Initiative,” which resulted to early initiation of ART to all people living with HIV regardless of CD4 cell count, or age, [35–37]. In 2017, for example, post the adoption of universal ART, 86% and 76% of adults and children below the age of 15 years, respectively, living with HIV were under the treatment programme [36].

Figure S5: Map of the Kingdom of Eswatini showing the four administrative regions, their areas, number of constituencies, population, and HIV prevalence. (Modified with permission from Attribution-NoDerivatives 4.0 International).

In addition to high HIV/AIDs prevalence, Eswatini has limited water supply and poor sanitation services [38]. Most water resources in Eswatini are shared with the neighbouring countries. For example, it shares five major rivers with South Africa including the Komati, Lomati, Pongola, Usuthu and Ngwavuma. The Komati and Lomati rivers, in the northern part of the country – both originate from South Africa and flow through Eswatini into Mozambique (Figure S6) [39]. Relatively irrigated croplands utilises some 870 Mm3 of water annually, with two-thirds in the sugarcane industry [40]. In addition, the Mbuluzi River originates in Eswatini and flows into Mozambique; whereas the Usuthu River originates in South Africa and flows via Eswatini into Mozambique [41]. Lastly, the Ngwavuma river, in the southern part of the country starts from Eswatini then flows into South Africa, and empties its waters in Mozambique [42]. This results in constraints in terms of water availability both for domestic and industrial purposes in Eswatini linked to shared transboundary water resources.

Figure S6: Water resources map of Eswatini detailing river basins [40].

Other water sources in the country are nine dams with total capacity of approximately 5.85 × 108 m3 [40]. Seven dams are used for sugarcane irrigation, one for hydroelectricity power production [43], and the larger dam (the Maguga Dam) is used for irrigation, hydroelectricity generation, and tourism purposes [44,45]. Although Eswatini has no substantial aquifers, groundwater is prominently used for domestic purposes, particularly in drier parts of the country [46] – with the largest portion of the country’s water resources used for the irrigation purposes.

UNICEF Eswatini report (2019) indicates that some 58% of the total population had access to basic sanitation services, 27% with limited services, 8% unimproved sanitation, and the rest from the remaining 7% constituted a runoff into the surface water systems [47]. The National Health and Hygiene Policy (2019) documented that 10 wastewater treatment plants (WWTPs) were operational in Eswatini [48] operated and monitored by Eswatini Water Services Corporation (EWSC). At a removal rate of 77.4% in the year 2018/2019 [49]. The total population connected to these systems are about 60 000 people [49]. Treatment technologies employed to treat generated influent includes waste stabilization ponds, vertical constructed wetlands (or reed beds), activated sludge, trickling filters, and chlorination [50]. The treated WWTP effluent is released into the river system, whilst the sludge is kept on sludge drying beds at the WWTP [51].

According to Sato et al.  [52], a significant portion of generated wastewater in SSA is released into the ecosystems untreated. This, in turn, can partly account for the spread of waterborne diseases e.g., diarrhoea and cholera in the SSA [52]. Based on the preceding facts, the lack of proper sanitation systems, high HIV/AIDS prevalence, the remarkable success of the ART programme [53], it is certain ARVs are present in the environment. Regardless of the absence of literature reporting on the quantification of ARVs in the environment in Eswatini, unlike in other SSA countries e.g., South Africa, Kenya and Zambia [2,14,16,19,21,54]. Furthermore, the country being the first to achieve the 90-90-90 and 95-95-95 paradigm motivates the need for systematic risk assessment of ARV moieties in Eswatini. Hence, in the next sections, we describe the approaches applied to quantify the potential risks of ARVs in the aquatic environments in Eswatini, where specific country data were taken into consideration.

4. Model equations

4.1 Quantification of ARV flows in wastewater treatment plants

As WWTP services are mostly accessible in the urban regions, the estimated masses of ARVs in the influent from all four regions of Eswatini for individual ARVs were computed using the expression:

MARVi, WWTPin=MARVSi × ARWWTP 					 	     	        	         (1)

where MARVi, WWTPin is the mass of ARVi in the influent wastewater (kg) for each region, and ARWWTP is the ratio of population connected to WWTPs in a particular region. Data used for the model was from the Eswatini annual vulnerability assessment and analysis report [55].

Conventional WWTPs generally have low efficacy in the removal of pharmaceuticals from wastewater treatment systems [56,57]. Different ARVs have high variation of removal efficiencies based on literature-documented values. The accessible efficiencies data were used to determine the geometric mean (GM) removal for each drug. Thereafter, the calculated GM was based on corresponding removal rates of a particular ARV for values reported in WWTPs in the African continent. Negative removal and zero removal rates were not considered as the geometric mean can only be computed using positive values. The computed GM efficiency was then used to calculate the removal of different ARVs considered in this work:

MARVi,  WWTPe = MARVi,WWTPin ×(1-GMremoval)	      						         (2)

where MARVi, WWTPe is the mass of ARVi in WWTP effluent in kg and GM removal is the geometric mean removal rate (%) of a particular ARVi in wastewater for each region.

4.2 Quantification of ARV flows in surface water

Low WWTP efficacies implies the release of pollutants into surface water systems. In Eswatini approximately seven percent of the population have no access to sanitation services (the Lubombo region had the highest runoff percentage), and therefore, ARVs are released in runoff into surface waters. The pollutants removed during the WWTPs are then accumulated onto sludge [58–60]. In most SSA countries wastewater receives minimal treatment [52], and the generated sludge are poorly managed with certain portions ending up in surface waters. In Eswatini, sludge is stored on sludge drying beds [51], and none used for agricultural purposes, hence the reason it is not accounted for in the calculations. The total estimated masses of ARVi in surface water were estimated using equation 3.

MARVi,  sw =(MARVSi, rural  ×%RO)+(MARVi,  WWTPi-MARVi,  WWTPe)					         (3)

where MARVi, sw is the flow of individual ARVi released into surface water (kg) and %RO is the percentage of runoff in Eswatini.

4.3 Exposure assessment

Previously, MECs of ARVs have been used to estimate their potential risks of ARVs in the aquatic systems [61,62]. However, the MEC values are based on limited sampling sites, and therefore, inadequate to quantify ARVs risk at a country level. Herein, predicted environmental concentrations (PECs) estimated using material flow analysis (MFA) approach [63] due to lack of data on the occurrence of ARVs in the aquatic environment in Eswatini. The PEC for both wastewater and surface water systems were determined. Hence, the PECs in wastewater (WW) were determined using the expressions:

PECi,WWTPe= MARVi, WWTPe  × 109WWt  								     (4)

PECi,wwTPi= MARVi, WWTPi  × 109WWt							                 (5)

where PECi, WWTPe is the concentration of specific ARVs in WWTP effluent (µg/L), PECi, WWTPi is the concentration of specific ARVs in WWTP influent (µg/L) and WWt is the fraction of  wastewater for a particular region (in litres) sourced from the Food and Agriculture Organization (FAO) [64]. The PECs in surface water were calculated using equation:

PECi,sw= MARVi,sw × 109WWut × DF 								                   (6)

Where PECi, sw is the concentration of individual ARVs in surface water in µg/L, WWut is the fraction of  untreated wastewater for each region, also obtained from the FAO [64] and DF is the dilution factor. Eswatini is an arid country with a long-term average annual rainfall range of 400–1 200 mm [26,65]. Our calculations were based on the year 2017, as this was the year with the most recent available data on the number of patients under the ART programme in Eswatini. Additionally, this was prior to the declaration by the WHO and UNAIDS of October 2015 [66] for universal treatment of all patients with HIV/AIDS. Thus, would yield masses for the period before the implementation of the new policy.  In this study, we used a DF = 3 as is the case in a study of Musee [67] for risk assessment of nanomaterials Johannesburg Metropolitan in South Africa.

4.4 Hazard characterisation

PNECs were estimated using either the half-maximal effective concentration (EC50) and/or lethal dose concentration (LC50) data values. The E(L)50 values were based on acute and/or chronic toxicity tests on three taxonomic groups (fish, daphnia and/or algae), and by use of an assessment factor (AF) [68–71] as defined by the European Medicines Agency (EMA) [72]. PNECs were determined using the expression below:

PNEC=E(L)C50AF 			    						    	       (7)

The AF used for acute toxicity to calculate the PNEC  is 1000 ECB, 2003), and the same value applies for toxicity data derived using the Ecological Structural Activity Relationships (ECOSAR) model [73]. This is because ECOSAR toxicity data are highly uncertain [74]. Due to lack of experimental ecotoxicity data for most ARVs in the aquatic environment. Therefore,  E(L)C50 values used in this study were derived using the ECOSAR v2.0 and EPI Suite software of the Environmental Protection Agency in the United States [75].

The ECOSAR model results indicated certain ARVs had one or more transformation products (TPs). Therefore, individual E(L)C50 values of the TPs for a given ARV derived from the ECOSAR model were used to calculate the geometric mean (GM) toxicity for each trophic level and drug [76–78]. The use of GM value was due to lack of a precise ratio of TPs for a given ARVs as are dependent on numerous environmental factors (e.g., electromagnetic radiation, dissolved organic carbon, temperature, and pH) [79–81]. For this reason, a single value could not be considered reflective of actual representation of TPs formed in the environment. For this reason, the GM toxicity value was considered as more representative.

5. Use of ARVs and their properties

Table S5: ARVs administered in the different regimes in Eswatini and their excretion rates.

| Regime | Drug | Symbol | Dosage (mg/d) | Drug use probability | Excretion rate (%) and references |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 1st line | Tenofovir | TDF | 300 | 0.416 |  | 85 | [82] |
|  | Lamivudine | 3TC | 300 | 0.750 |  | 75 | [83] |
|  | Efavirenz | EFV | 600 | 0.469 |  | 62 | [84] |
|  | Nevirapine | NVP | 400 | 0.285 |  | 2.7 | [85] |
|  | Zidovudine | AZT | 600 | 0.338 |  | 75 | [86,87] |
|  | Dolutegravir | DTG | 50 | 0.02 |  | 95.6 | [88] |
| 2nd line | Lopinavir | LPV | 800 | 0.02 |  | 21 | [89] |
|  | Didanosine | DDI | 800 | 0.02 |  | 12.4 | [90] |
|  | Atazanavir | ATV | 300 | 0.02 |  | 7 | [91] |
|  | Ritonavir | RTV | 100 | 0.02 |  | 97.6 | [92] |

Drug regimens and administered drug dosages sourced from the Ministry of Health (MoH) in Eswatini

Table S6: Geometric mean removal rates of ARV drugs considered in this work

| ARV Drug | Influent | Effluent | Removal rate | Geometric mean | Reference |
| --- | --- | --- | --- | --- | --- |
| Atazanavir | 1.4 | 0.74 | 47.1 | 47.1 | [2] |
| Efavirenz | 0.78 | 0.11 | 85.9 | 45.0 | [6] |
|  | 0.46 | 0.1 | 78.3 |  | [6] |
|  | 1.02 | 0.11 | 89.2 |  | [6] |
|  | 34 | 20 | 41.2 |  | [2] |
|  | 34 | 33 | 2.9 |  | [2] |
|  | 24 | 33 | -37.5 |  | [2] |
|  | 1.37 | 3.63 | -165.0 |  | [8] |
|  | 26.3 | 37.3 | -41.8 |  | [8] |
|  | 1.67 | 3.27 | -95.8 |  | [8] |
|  | 2.22 | 3.64 | -64.0 |  | [8] |
|  | 17.4 | 7.1 | 59.2 |  | [9] |
|  | 15.4 | 1.93 | 87.5 |  | [10] |
| Lamivudine | 2.2 | 0.13 | 94.1 | 53.8 | [2] |
|  | 30.3 | 19.9 | 34.3 |  | [6] |
|  | 60.68 | 31.07 | 48.8 |  | [6] |
|  | 118.97 | 55.76 | 53.1 |  | [13] |
| Lopinavir | 1.3 | 3.8 | -192.3 | - | [2] |
|  | 1.2 | 1.9 | -192.3 |  | [2] |
|  | 2.5 | 3.8 | -58.3 |  | [2] |
| Nevirapine | 0.85 | 1.03 | -21.18 | 20.1 | [6] |
|  | 3.3 | 2.11 | 36.06 |  | [6] |
|  | 2.08 | 2.03 | 2.40 |  | [6] |
|  | 0.67 | 0.54 | 19.40 |  | [2] |
|  | 2.1 | 1.9 | 9.52 |  | [2] |
|  | 2.8 | 1.4 | 50.00 |  | [2] |
|  | 0.681 | 0.764 | -12.19 |  | [10] |
|  | 2.1 | 0.35 | 83.33 |  | [9] |
|  | 0.68 | 1.72 | -152.94 |  | [13] |
| Ritonavir | 3.2 | 1.5 | 53.1 | 54.7 | [2] |
|  | 1.6 | 0.91 | 43.1 |  | [2] |
|  | 1.6 | 0.46 | 71.3 |  | [2] |
| Tenofovir | 0.25 | 0.17 | 32.00 | 32.0 | [8] |
| Zidovudine | 107 | 97.7 | 8.7 | 53.5 | [10] |
|  | 6.9 | 0.087 | 98.7 |  | [2] |
|  | 53 | 0.5 | 99.1 |  | [2] |
|  | 11 | 0.43 | 96.1 |  | [2] |

Table S7: Number of people on various ART regimes in Sub-Saharan African countries.

| Year, Country | Regime | Drug combination | % people/ regime | Reference |
| --- | --- | --- | --- | --- |
| 2017 Eswatini | 1st line | TDF + 3TC + EFV | 39.9 | [93] |
|  |  | TDF + 3TC + NVP | 1.7 |  |
|  |  | AZT + 3TC + NVP | 26.8 |  |
|  |  | AZT + 3TC + EFV | 7 |  |
|  | 2nd line | AZT + 3TC + LPV/r | 6.5 |  |
|  |  | AZT + 3TC + ATV/r |  |  |
| 2020 South Africa | 1st line | TDF + FTC + EFV | 98.9 | [94] |
|  | 2nd line | AZT + 3TC + LPV/r | 0.06 |  |
|  | 3rd line | AZT + 3TC + DTG | 0.0003 |  |
|  |  | TDF + FTC + DTG |  |  |
| 2016 South Africa | 1st line | TDF + FTC + EFV | 95.7 | [95] |
|  | 2nd line | AZT + 3TC + LPV/r | 0.04 |  |
|  | 3rd line | AZT + 3TC + DTG | 0.0002 |  |
|  |  | TDF + FTC + DTG |  |  |
| 2009 Malawi | 1st line | d4T + 3TC + NVP | 99% | [96] |
|  |  | AZT + 3TC + NVP |  |  |
|  |  | AZT + 3TC + EFZ |  |  |
|  | 2nd line | AZT + 3TC + TDF/ LPV/r | < 1% |  |

Table S8: Properties of ARV drugs considered in this study

| ARV drug | CAS number | Molecular formula | Molecular structure | Log Kow | Water solubility (mg/L) |
| --- | --- | --- | --- | --- | --- |
| Atazanavir | 198904-31-3 | C38H52N6O7 |  | 2.88 | 0.11 |
| Efavirenz | 154598-52-4 | C14H9ClF3NO2 |  | 4.7 | 0.093 |
| Lamivudine | 134678-17-4 | C8H11N3O3S |  | -9.54 | 70 000 |
| Lopinavir | 192725-17-0 | C37H48N4O5 |  | 5.94 | 0.0077 |
| Didanosine | 69655-05-6 | C10H12N4O3 |  | -1.24 | 15 800 |
| Nevirapine | 129618-40-2 | C15H14N4O |  | 3.89 | 100 |
| Ritonavir | 155213-67-5 | C37H48N6O5S2 |  | 6.27 | 0.00011 |
| Tenofovir | 147127-20-6 | C9H14N5O4P |  | -1.6 | 13 400 |
| Dolutegravir | 1051375-16-6 | C20H19F2N3O5 |  | 1.62 | 95 |
| Zidovudine | 30516-87-1 | C10H13N5O4 |  | 0.05 | 20100 |

All information sourced from https://pubchem.ncbi.nlm.nih.gov/ Accessed: 17 February 2022

References

[1]	UNAIDS, Global HIV & AIDS statistics — Fact sheet, (2021). https://www.unaids.org/en/resources/fact-sheet (accessed June 28, 2022).

[2]	O.A. Abafe, J. Späth, J. Fick, S. Jansson, C. Buckley, A. Stark, B. Pietruschka, B.S. Martincigh, LC-MS/MS determination of antiretroviral drugs in influents and effluents from wastewater treatment plants in KwaZulu-Natal, South Africa, Chemosphere. 200 (2018) 660–670. https://doi.org/10.1016/j.chemosphere.2018.02.105.

[3]	J. Funke, C. Prasse, T.A. Ternes, Identification of transformation products of antiviral drugs formed during biological wastewater treatment and their occurrence in the urban water cycle, Water Res. 98 (2016) 75–83. https://doi.org/10.1016/j.watres.2016.03.045.

[4]	L. Boulard, G. Dierkes, T. Ternes, Utilization of large volume zwitterionic hydrophilic interaction liquid chromatography for the analysis of polar pharmaceuticals in aqueous environmental samples: Benefits and limitations, J. Chromatogr. A. 1535 (2018) 27–43. https://doi.org/10.1016/j.chroma.2017.12.023.

[5]	C. Prasse, M.P. Schlüsener, R. Schulz, T.A. Ternes, Antiviral Drugs in Wastewater and Surface Waters: A New Pharmaceutical Class of Environmental Relevance?, Environ. Sci. Technol. 44 (2010) 1728–1735. https://doi.org/10.1021/es903216p.

[6]	K.O. K’oreje, L. Vergeynst, D. Ombaka, P. De Wispelaere, M. Okoth, H. Van Langenhove, K. Demeestere, Occurrence patterns of pharmaceutical residues in wastewater, surface water and groundwater of Nairobi and Kisumu city, Kenya, Chemosphere. 149 (2016) 238–244. https://doi.org/10.1016/j.chemosphere.2016.01.095.

[7]	S.P. Mtolo, P.N. Mahlambi, L.M. Madikizela, Synthesis and application of a molecularly imprinted polymer in selective solid-phase extraction of efavirenz from water, Water Sci. Technol. 79 (2019) 356–365. https://doi.org/10.2166/wst.2019.054.

[8]	N.Y. Mlunguza, S. Ncube, P.N. Mahlambi, L. Chimuka, L.M. Madikizela, Determination of selected antiretroviral drugs in wastewater, surface water and aquatic plants using hollow fibre liquid phase microextraction and liquid chromatography - tandem mass spectrometry, J. Hazard. Mater. 382 (2020) 121067. https://doi.org/10.1016/j.jhazmat.2019.121067.

[9]	C. Schoeman, M. Mashiane, M. Dlamini, O.J. Okonkwo, Quantification of Selected Antiretroviral Drugs in a Wastewater Treatment Works in South Africa Using GC-TOFMS, J. Chromatogr. Sep. Tech. 06 (2015). https://doi.org/10.4172/2157-7064.1000272.

[10]	T.T. Mosekiemang, M.A. Stander, A. de Villiers, Simultaneous quantification of commonly prescribed antiretroviral drugs and their selected metabolites in aqueous environmental samples by direct injection and solid phase extraction liquid chromatography - tandem mass spectrometry, Chemosphere. 220 (2019) 983–992. https://doi.org/10.1016/j.chemosphere.2018.12.205.

[11]	K.O. K’oreje, F.J. Kandie, L. Vergeynst, M.A. Abira, H. Van Langenhove, M. Okoth, K. Demeestere, Occurrence, fate and removal of pharmaceuticals, personal care products and pesticides in wastewater stabilization ponds and receiving rivers in the Nzoia Basin, Kenya, Sci. Total Environ. 637–638 (2018) 336–348. https://doi.org/10.1016/j.scitotenv.2018.04.331.

[12]	E. Ngumba, A. Gachanja, T. Tuhkanen, Occurrence of selected antibiotics and antiretroviral drugs in Nairobi River Basin, Kenya, Sci. Total Environ. 539 (2016) 206–213. https://doi.org/10.1016/j.scitotenv.2015.08.139.

[13]	E. Ngumba, A. Gachanja, J. Nyirenda, J. Maldonado, T. Tuhkanen, Occurrence of antibiotics and antiretroviral drugs in source-separated urine, groundwater, surface water and wastewater in the peri-urban area of Chunga in Lusaka, Zambia, Water SA. 46 (2020) 278–284.

[14]	T.P. Wood, C.S.J. Duvenage, E. Rohwer, The occurrence of anti-retroviral compounds used for HIV treatment in South African surface water, Environ. Pollut. 199 (2015) 235–243. https://doi.org/10.1016/j.envpol.2015.01.030.

[15]	J. Margot, C. Kienle, A. Magnet, M. Weil, L. Rossi, L.F. de Alencastro, C. Abegglen, D. Thonney, N. Chèvre, M. Schärer, D.A. Barry, Treatment of micropollutants in municipal wastewater: Ozone or powdered activated carbon?, Sci. Total Environ. 461–462 (2013) 480–498. https://doi.org/10.1016/j.scitotenv.2013.05.034.

[16]	E. Ngumba, Occurrence and control of selected antibiotics and antiretroviral drugs in urban hydrological cycles, (2018) 95.

[17]	Y. Aminot, X. Litrico, M. Chambolle, C. Arnaud, P. Pardon, H. Budzindki, Development and application of a multi-residue method for the determination of 53 pharmaceuticals in water, sediment, and suspended solids using liquid chromatography-tandem mass spectrometry, Anal. Bioanal. Chem. 407 (2015) 8585–8604. https://doi.org/10.1007/s00216-015-9017-3.

[18]	T.P. Wood, C.S.J. Duvenage, E. Rohwer, The occurrence of anti-retroviral compounds used for HIV treatment in South African surface water, Environ. Pollut. 199 (2015) 235–243. https://doi.org/10.1016/j.envpol.2015.01.030.

[19]	K.O. K’oreje, K. Demeestere, P. De Wispelaere, L. Vergeynst, J. Dewulf, H. Van Langenhove, From multi-residue screening to target analysis of pharmaceuticals in water: Development of a new approach based on magnetic sector mass spectrometry and application in the Nairobi River basin, Kenya, Sci. Total Environ. 437 (2012) 153–164. https://doi.org/10.1016/j.scitotenv.2012.07.052.

[20]	I.J. Fisher, P.J. Phillips, K.M. Colella, S.C. Fisher, T. Tagliaferri, W.T. Foreman, E.T. Furlong, The impact of onsite wastewater disposal systems on groundwater in areas inundated by Hurricane Sandy in New York and New Jersey, Mar. Pollut. Bull. 107 (2016) 509–517. https://doi.org/10.1016/j.marpolbul.2016.04.038.

[21]	C. Rimayi, D. Odusanya, J.M. Weiss, J. de Boer, L. Chimuka, Contaminants of emerging concern in the Hartbeespoort Dam catchment and the uMngeni River estuary 2016 pollution incident, South Africa, Sci. Total Environ. 627 (2018) 1008–1017. https://doi.org/10.1016/j.scitotenv.2018.01.263.

[22]	E.T. Furlong, A.L. Batt, S.T. Glassmeyer, M.C. Noriega, D.W. Kolpin, H. Mash, K.M. Schenck, Nationwide reconnaissance of contaminants of emerging concern in source and treated drinking waters of the United States: Pharmaceuticals, Sci. Total Environ. 579 (2017) 1629–1642. https://doi.org/10.1016/j.scitotenv.2016.03.128.

[23]	L. Boulard, G. Dierkes, T. Ternes, Utilization of large volume zwitterionic hydrophilic interaction liquid chromatography for the analysis of polar pharmaceuticals in aqueous environmental samples: Benefits and limitations, J. Chromatogr. A. 1535 (2018) 27–43. https://doi.org/10.1016/j.chroma.2017.12.023.

[24]	J. Giebułtowicz, S. Tyski, R. Wolinowska, W. Grzybowska, T. Zaręba, A. Drobniewska, P. Wroczyński, G. Nałęcz-Jawecki, Occurrence of antimicrobial agents, drug-resistant bacteria, and genes in the sewage-impacted Vistula River (Poland), Environ. Sci. Pollut. Res. 25 (2018) 5788–5807. https://doi.org/10.1007/s11356-017-0861-x.

[25]	A.G. Sam, B.O. Abidoye, S. Mashaba, Climate change and household welfare in sub-Saharan Africa: empirical evidence from Swaziland, Food Secur. 13 (2021) 439–455. https://doi.org/10.1007/s12571-020-01113-z.

[26]	E.J. Mwendera, Available water for hydropower generation in Swaziland, Phys. Chem. Earth Parts ABC. 31 (2006) 952–959. https://doi.org/10.1016/j.pce.2006.08.015.

[27]	World Bank, Rural population (% of total population) - Eswatini | Data, (2020). https://data.worldbank.org/indicator/SP.RUR.TOTL.ZS?locations=SZ (accessed January 14, 2022).

[28]	Eswatini Population 2021 (Demographics, Maps, Graphs), (n.d.). https://worldpopulationreview.com/countries/eswatini-population (accessed October 19, 2021).

[29]	M. Mangaliso, T. Dlamini, Predictors of food insecurity in Eswatini : lessons from the 2015/16 El Niño induced drought, Afr. Rev. Econ. Finance. 10 (2018) 69–96. https://doi.org/10.10520/EJC-1297b2d32e.

[30]	UNFPA, Swaziland Releases Population Count from 2017 Census, UNFPA Kingd. Eswatini. (2017). https://eswatini.unfpa.org/en/news/swaziland-releases-population-count-2017-census (accessed October 17, 2022).

[31]	S. Horter, S. Bernays, Z. Thabede, V. Dlamini, B. Kerschberger, M. Pasipamire, B. Rusch, A. Wringe, “I don’t want them to know”: how stigma creates dilemmas for engagement with Treat-all HIV care for people living with HIV in Eswatini, Afr. J. AIDS Res. 18 (2019) 27–37. https://doi.org/10.2989/16085906.2018.1552163.

[32]	D. MacKellar, D. Williams, B. Bhembe, M. Dlamini, J. Byrd, L. Dube, S. Mazibuko, T. Ao, I. Pathmanathan, A.F. Auld, P. Faura, N. Lukhele, C. Ryan, Peer-Delivered Linkage Case Management and Same-Day ART Initiation for Men and Young Persons with HIV Infection — Eswatini, 2015–2017, Morb. Mortal. Wkly. Rep. 67 (2018) 663–667. https://doi.org/10.15585/mmwr.mm6723a3.

[33]	J. Justman, J.B. Reed, G. Bicego, D. Donnell, K. Li, N. Bock, A. Koler, N.M. Philip, C.K. Mlambo, B.S. Parekh, Y.T. Duong, D.L. Ellenberger, W.M. El-Sadr, R. Nkambule, Swaziland HIV Incidence Measurement Survey (SHIMS): a prospective national cohort study, Lancet HIV. 4 (2017) e83–e92. https://doi.org/10.1016/S2352-3018(16)30190-4.

[34]	World Health Organization, Consolidated guidelines on the use of antiretroviral drugs for treating and preventing HIV infection: recommendations for a public health approach, 2nd ed, World Health Organization, Geneva, 2016. https://apps.who.int/iris/handle/10665/208825 (accessed February 24, 2023).

[35]	A. Akullian, M. Morrison, G.P. Garnett, Z. Mnisi, N. Lukhele, D. Bridenbecker, A. Bershteyn, The effect of 90-90-90 on HIV-1 incidence and mortality in eSwatini: a mathematical modelling study, Lancet HIV. 7 (2020) e348–e358. https://doi.org/10.1016/S2352-3018(19)30436-9.

[36]	C. Chouraya, K. Ashburn, P. Khumalo, L. Mpango, N. Mthethwa, R. Machekano, L. Guay, L.M. Mofenson, Association of Antiretroviral Drug Regimen With Viral Suppression in HIV-positive Children on Antiretroviral Therapy in Eswatini, Pediatr. Infect. Dis. J. 38 (2019) 835. https://doi.org/10.1097/INF.0000000000002347.

[37]	C.P. Dlamini, M. Matlakala, Disclosure of human immunodeficiency virus status to children : pattern followed by parents and caregivers, Afr. J. Prim. Health Care Fam. Med. 12 (2020) 1–6. https://doi.org/10.4102/phcfm.v12i1.2230.

[38]	B. Howard, K. Han, African governments failing in provision of water and sanitation, majoring of citizens say, (2020). https://www.afrobarometer.org/wp-content/uploads/migrated/files/publications/Dispatches/ab_r7_dispatchno349_pap14_water_and_sanitation_in_africa.pdf (accessed July 9, 2022).

[39]	J.I. Matondo, K.M. Msibi, Estimation of the Impact of Climate Change on Hydrology and Water Resources in Swaziland, Water Int. 26 (2001) 425–434. https://doi.org/10.1080/02508060108686934.

[40]	P. Mlilo, B. Mhlanga, A. Senzanje, Emerging Issues in Water Resources Management in Swaziland, (2008).

[41]	A.L. Breverman, J.E. Helminiak, S.M. England, Water Security in Eswatini, Africa, (2020) 64–72. https://doi.org/10.1061/9780784482940.007.

[42]	P. Smedley, Groundwater quality: eSwatini, (2021). https://nora.nerc.ac.uk/id/eprint/530838/ (accessed July 9, 2022).

[43]	N. Mashwama, T. Shongwe, Proposing the best combination of Renewable Energy resources available to the country of Eswatini, in: 2020 Int. Conf. Artif. Intell. Big Data Comput. Data Commun. Syst. IcABCD, 2020: pp. 1–8. https://doi.org/10.1109/icABCD49160.2020.9183867.

[44]	E.A.K. Kalitsi, PROBLEMS AND PROSPECTS FOR HYDROPOWER DEVELOPMENT IN AFRICA, (2003) 25.

[45]	A.M. Manyatsi, R. Brown, iwrm-report_swaziland.pdf, (2009). https://www.gwp.org/globalassets/global/gwp-saf-files/iwrm-report_swaziland.pdf (accessed October 5, 2021).

[46]	N.S. Robins, A review of groundwater resources and well yields in Swaziland, (1980) 4.

[47]	UNICEF, Water, sanitation and hygiene (WASH), (2019). https://www.unicef.org/eswatini/water-sanitation-and-hygiene-wash (accessed October 6, 2021).

[48]	EMoH, Eswatini National Sanitation Policy-report-2019.pdf.pdf, (2019). https://www.unicef.org/eswatini/media/876/file/Eswatini%20National%20Sanitation%20Policy-report-2019.pdf.pdf (accessed July 9, 2022).

[49]	EWSC, Eswatini Water Services Corporation 2018 Annual Report.pdf, (2018).

[50]	J. Mtshali, A. Fadiran, Characterization of Sewage Sludge Generated from Wastewater Treatment Plants in Swaziland in Relation to Agricultural Uses, Resour. Environ. 4 (2014) 190–199. https://doi.org/10.5923/j.re.20140404.02.

[51]	A.T. Tiruneh, J. Mtshali, A. Fadiran, Characterization of Sewage Sludge Generated from Wastewater Treatment Plants in Swaziland in Relation to Agricultural Uses, Resour. Environ. 4 (2014) 190–199. https://doi.org/10.5923/j.re.20140404.02.

[52]	T. Sato, M. Qadir, S. Yamamoto, T. Endo, A. Zahoor, Global, regional, and country level need for data on wastewater generation, treatment, and use, Agric. Water Manag. 130 (2013) 1–13. https://doi.org/10.1016/j.agwat.2013.08.007.

[53]	S. Ncube, L.M. Madikizela, L. Chimuka, M.M. Nindi, Environmental fate and ecotoxicological effects of antiretrovirals: A current global status and future perspectives, Water Res. 145 (2018) 231–247. https://doi.org/10.1016/j.watres.2018.08.017.

[54]	C. Swanepoel, H. Bouwman, R. Pieters, C. Bezuidenhout, Presence, concentrations and potential implications of HIV-ARVs in selected water sources in South Africa, 2015. https://doi.org/10.13140/RG.2.2.20637.51688.

[55]	Eswatini VAC, ANNUAL VULNERABILITY ASSESSMENT & ANALYSIS REPORT 2018, (2018). https://www.ipcinfo.org/fileadmin/user_upload/ipcinfo/docs/Eswatini_VAC_Annual_Assessment_Report2018.pdf (accessed October 19, 2022).

[56]	L.M. Madikizela, L. Chimuka, Occurrence of naproxen, ibuprofen, and diclofenac residues in wastewater and river water of KwaZulu-Natal Province in South Africa, Environ. Monit. Assess. 189 (2017) 348. https://doi.org/10.1007/s10661-017-6069-1.

[57]	B. Moslah, E. Hapeshi, A. Jrad, D. Fatta-Kassinos, A. Hedhili, Pharmaceuticals and illicit drugs in wastewater samples in north-eastern Tunisia, Environ. Sci. Pollut. Res. 25 (2018) 18226–18241. https://doi.org/10.1007/s11356-017-8902-z.

[58]	M.S. Díaz-Cruz, D. Barceló, Trace organic chemicals contamination in ground water recharge, Chemosphere. 72 (2008) 333–342. https://doi.org/10.1016/j.chemosphere.2008.02.031.

[59]	B. Halling-Sørensen, S. Nors Nielsen, P.F. Lanzky, F. Ingerslev, H.C. Holten Lützhøft, S.E. Jørgensen, Occurrence, fate and effects of pharmaceutical substances in the environment- A review, Chemosphere. 36 (1998) 357–393. https://doi.org/10.1016/S0045-6535(97)00354-8.

[60]	L.M. Madikizela, N.T. Tavengwa, L. Chimuka, Status of pharmaceuticals in African water bodies: Occurrence, removal and analytical methods, J. Environ. Manage. 193 (2017) 211–220. https://doi.org/10.1016/j.jenvman.2017.02.022.

[61]	C.W. Muriuki, P.G. Home, J.M. Raude, E.K. Ngumba, G.K. Munala, P.K. Kairigo, A.N. Gachanja, T.A. Tuhkanen, Occurrence, distribution, and risk assessment of pharmerciuticals in wastewater and open surface drains of peri-urban areas: Case study of Juja town, Kenya, Environ. Pollut. 267 (2020) 115503. https://doi.org/10.1016/j.envpol.2020.115503.

[62]	E. Ngumba, A. Gachanja, T. Tuhkanen, Occurrence of selected antibiotics and antiretroviral drugs in Nairobi River Basin, Kenya, Sci. Total Environ. 539 (2016) 206–213. https://doi.org/10.1016/j.scitotenv.2015.08.139.

[63]	B. Nowack, Evaluation of environmental exposure models for engineered nanomaterials in a regulatory context, NanoImpact. 8 (2017) 38–47. https://doi.org/10.1016/j.impact.2017.06.005.

[64]	AQUASTAT, AQUASTAT database Database Query Results, (2021). https://www.fao.org/aquastat/statistics/query/results.html (accessed June 28, 2022).

[65]	A.M. Manyatsi, N. Zwane, M. Dlamini, Evaluation of Satellite Rainfall Estimates for Swaziland, Am. J. Agric. For. 3 (2015) 93. https://doi.org/10.11648/j.ajaf.20150303.15.

[66]	USAID, Test and Start: Optimizing Antiretroviral Therapy Services to Save Lives, (2019). https://2017-2020.usaid.gov/global-health/health-areas/hiv-and-aids/technical-areas/test-and-start (accessed May 2, 2023).

[67]	N. Musee, Simulated environmental risk estimation of engineered nanomaterials: A case of cosmetics in Johannesburg City, Hum. Exp. Toxicol. 30 (2011) 1181–1195. https://doi.org/10.1177/0960327110391387.

[68]	M.D. Hernando, M. Mezcua, A.R. Fernández-Alba, D. Barceló, Environmental risk assessment of pharmaceutical residues in wastewater effluents, surface waters and sediments, Talanta. 69 (2006) 334–342. https://doi.org/10.1016/j.talanta.2005.09.037.

[69]	N. Chèvre, S. Coutu, J. Margot, H.K. Wynn, H.-P. Bader, R. Scheidegger, L. Rossi, Substance flow analysis as a tool for mitigating the impact of pharmaceuticals on the aquatic system, Water Res. 47 (2013) 2995–3005. https://doi.org/10.1016/j.watres.2013.03.004.

[70]	C.I. Kosma, D.A. Lambropoulou, T.A. Albanis, Investigation of PPCPs in wastewater treatment plants in Greece: Occurrence, removal and environmental risk assessment, Sci. Total Environ. 466–467 (2014) 421–438. https://doi.org/10.1016/j.scitotenv.2013.07.044.

[71]	E. Molnar, G. Maasz, Z. Pirger, Environmental risk assessment of pharmaceuticals at a seasonal holiday destination in the largest freshwater shallow lake in Central Europe, Environ. Sci. Pollut. Res. (2020). https://doi.org/10.1007/s11356-020-09747-4.

[72]	EMA, Environmental risk assessment of medicinal products for human use, Eur. Med. Agency. (2018). https://www.ema.europa.eu/en/environmental-risk-assessment-medicinal-products-human-use (accessed September 23, 2021).

[73]	M. Papageorgiou, C. Kosma, D. Lambropoulou, Seasonal occurrence, removal, mass loading and environmental risk assessment of 55 pharmaceuticals and personal care products in a municipal wastewater treatment plant in Central Greece, Sci. Total Environ. 543 (2016) 547–569. https://doi.org/10.1016/j.scitotenv.2015.11.047.

[74]	Y. Zhang, T. Zhang, C. Guo, J. Lv, Z. Hua, S. Hou, Y. Zhang, W. Meng, J. Xu, Drugs of abuse and their metabolites in the urban rivers of Beijing, China: Occurrence, distribution, and potential environmental risk, Sci. Total Environ. 579 (2017) 305–313. https://doi.org/10.1016/j.scitotenv.2016.11.101.

[75]	US EPA, EPI SuiteTM-Estimation Program Interface, (2012). https://www.epa.gov/tsca-screening-tools/epi-suitetm-estimation-program-interface (accessed June 29, 2022).

[76]	P.M. Chapman, Including or excluding toxicity test data for development of a geometric mean: Letter to the Editor, Environ. Toxicol. Chem. 34 (2015) 1691–1692. https://doi.org/10.1002/etc.3012.

[77]	B.G. Church, P.A. Van Sprang, M.J. Chowdhury, D.K. DeForest, Updated species sensitivity distribution evaluations for acute and chronic lead toxicity to saltwater aquatic life: Saltwater lead toxicity to aquatic life, Environ. Toxicol. Chem. 36 (2017) 2974–2980. https://doi.org/10.1002/etc.3863.

[78]	T.H. Hutchinson, J. Solbe, P.J. Kloepper-Sams, Analysis of the ecetoc aquatic toxicity (EAT) database III — Comparative toxicity of chemical substances to different life stages of aquatic organisms, Chemosphere. 36 (1998) 129–142. https://doi.org/10.1016/S0045-6535(97)10025-X.

[79]	R. Osawa, B. T. Barrocas, O. C. Monteiro, M.C. Oliveira, M.H. Florêncio, Photocatalytic degradation of cyclophosphamide and ifosfamide: Effects of wastewater matrix, transformation products and in silico toxicity prediction, Sci. Total Environ. 692 (2019) 503–510. https://doi.org/10.1016/j.scitotenv.2019.07.247.

[80]	A. Timm, E. Borowska, M. Majewsky, S. Merel, C. Zwiener, S. Bräse, H. Horn, Photolysis of four β‑lactam antibiotics under simulated environmental conditions: Degradation, transformation products and antibacterial activity, Sci. Total Environ. 651 (2019) 1605–1612. https://doi.org/10.1016/j.scitotenv.2018.09.248.

[81]	C. Trautwein, K. Kümmerer, Degradation of the tricyclic antipsychotic drug chlorpromazine under environmental conditions, identification of its main aquatic biotic and abiotic transformation products by LC–MSn and their effects on environmental bacteria, J. Chromatogr. B. 889–890 (2012) 24–38. https://doi.org/10.1016/j.jchromb.2012.01.022.

[82]	A.B. Libório, L. Andrade, L.V.B. Pereira, T.R.C. Sanches, M.H. Shimizu, A.C. Seguro, Rosiglitazone reverses tenofovir-induced nephrotoxicity, Kidney Int. 74 (2008) 910–918. https://doi.org/10.1038/ki.2008.252.

[83]	A.E. Heald, P.H. Hsyu, G.J. Yuen, P. Robinson, P. Mydlow, J.A. Bartlett, Pharmacokinetics of lamivudine in human immunodeficiency virus-infected patients with renal dysfunction, Antimicrob. Agents Chemother. 40 (1996) 1514–1519. https://doi.org/10.1128/AAC.40.6.1514.

[84]	N.Y. Rakhmanina, J.N. van den Anker, Efavirenz in the therapy of HIV infection, Expert Opin. Drug Metab. Toxicol. 6 (2010) 95–103. https://doi.org/10.1517/17425250903483207.

[85]	P. Riska, M. Lamson, T. MacGregor, J. Sabo, S. Hattox, J. Pav, J. Keirns, Disposition and Biotransformation of the Antiretroviral Drug Nevirapine in Humans, Drug Metab. Dispos. 27 (1999) 895–901.

[86]	G.J. Veal, D.J. Back, Metabolism of zidovudine, Gen. Pharmacol. Vasc. Syst. 26 (1995) 1469–1475. https://doi.org/10.1016/0306-3623(95)00047-X.

[87]	J. Warren Beach, Chemotherapeutic agents for human immunodeficiency virus infection: mechanism of action, pharmacokinetics, metabolism, and adverse reactions, Clin. Ther. 20 (1998) 2–25. https://doi.org/10.1016/S0149-2918(98)80031-3.

[88]	S. Castellino, L. Moss, D. Wagner, J. Borland, I. Song, S. Chen, Y. Lou, S.S. Min, I. Goljer, A. Culp, S.C. Piscitelli, P.M. Savina, Metabolism, Excretion, and Mass Balance of the HIV-1 Integrase Inhibitor Dolutegravir in Humans, Antimicrob. Agents Chemother. 57 (2013) 3536–3546. https://doi.org/10.1128/AAC.00292-13.

[89]	R.S. Cvetkovic, K.L. Goa, Lopinavir/Ritonavir, Drugs. 63 (2003) 769–802. https://doi.org/10.2165/00003495-200363080-00004.

[90]	E. Singlas, A.M. Taburet, F. Borsa Lebas, O. Parent de Curzon, A. Sobel, P. Chauveau, B. Viron, R. al Khayat, J.L. Poignet, F. Mignon, G. Humbert, J.P. Fillastre, Didanosine pharmacokinetics in patients with normal and impaired renal function: influence of hemodialysis, Antimicrob. Agents Chemother. 36 (1992) 1519–1524. https://doi.org/10.1128/AAC.36.7.1519.

[91]	J. Milburn, R. Jones, J.B. Levy, Renal effects of novel antiretroviral drugs, Nephrol. Dial. Transplant. 32 (2017) 434–439. https://doi.org/10.1093/ndt/gfw064.

[92]	A. Hsu, G.R. Granneman, G. Witt, C. Locke, J. Denissen, A. Molla, J. Valdes, J. Smith, K. Erdman, N. Lyons, P. Niu, J.P. Decourt, J.B. Fourtillan, J. Girault, J.M. Leonard, Multiple-dose pharmacokinetics of ritonavir in human immunodeficiency virus-infected subjects, Antimicrob. Agents Chemother. (1997). https://doi.org/10.1128/AAC.41.5.898.

[93]	D. Etoori, I. Ciglenecki, M. Ndlangamandla, C.G. Edwards, K. Jobanputra, M. Pasipamire, G. Maphalala, C. Yang, I. Zabsonre, S.M. Kabore, J. Goiri, R. Teck, B. Kerschberger, Successes and challenges in optimizing the viral load cascade to improve antiretroviral therapy adherence and rationalize second-line switches in Swaziland, J. Int. AIDS Soc. 21 (2018) e25194. https://doi.org/10.1002/jia2.25194.

[94]	Private communication, ART Statistics in South Africa, (n.d.).

[95]	W.F. Venter, B. Kaiser, Y. Pillay, F. Conradie, G.B. Gomez, P. Clayden, M. Matsolo, C. Amole, L. Rutter, F. Abdullah, E.J. Abrams, C.P. Casas, M. Barnhart, A. Pillay, A. Pozniak, A. Hill, L. Fairlie, M. Boffito, M. Moorhouse, M. Chersich, C. Serenata, J. Quevedo, G. Loots, Cutting the cost of South African antiretroviral therapy using newer, safer drugs, S. Afr. Med. J. 107 (2016) 28. https://doi.org/10.7196/SAMJ.2017.v107i1.12058.

[96]	J. Luebbert, H. Tweya, S. Phiri, T. Chaweza, J. Mwafilaso, M.C. Hosseinipour, H. Ramroth, P. Schnitzler, F. Neuhann, Virological Failure and Drug Resistance in Patients on Antiretroviral Therapy After Treatment Interruption in Lilongwe, Malawi, Clin. Infect. Dis. 55 (2012) 441–448. https://doi.org/10.1093/cid/cis438.